FOENIX K2 & JR JR

NEW CORE X2 DEFINITIONS

IO CTRL

MMU SYSTEM FOR X2 CORES

BANKS IN X2 CORES

To Compose the SRAM Address

- Only the bit[5:0] are used in the MMU Output Value (for the selected slot),

- Then the new bit on Register $0002 + $0003 are added depending again on the slot # (this will work only with the Page 0)

So for an example of SRAM Address, let's say we used slot #1 and we want to access SRAM 512K Bank 1.

- The Value for address $0007 would be $01 (CPU Mem: $2000 - $3FFF (when page 0 is selected)

- Value for address $0002 would be $04

And that would give you access to : $08_2000 in absolute address, So bit[9:8] of $0002 or ($0003) + MMU Bit[5:0] + CPU Addy [12:0]

REGULAR MMU DECODING (NON X2 MODE)

VICKY CTRL REGISTERS CHANGES

MASTER_CTRL_REG_L = $D000

| Mstr_Ctrl_Text_Mode_En = $01 | Enable the Text Mode |
| --- | --- |
| Mstr_Ctrl_Text_Overlay = $02 | Enable the Overlay of the text mode on top of Graphic Mode (the Background Color is ignored) |
| Mstr_Ctrl_Graph_Mode_En = $04 | Enable the Graphic Mode |
| Mstr_Ctrl_Bitmap_En = $08 | Enable the Bitmap Module In Vicky |
| Mstr_Ctrl_TileMap_En = $10 | Enable the Tile Module in Vicky |
| Mstr_Ctrl_Sprite_En = $20 | Enable the Sprite Module in Vicky |
| Mstr_Ctrl_GAMMA_En = $40 | Enable the GAMMA correction - The Analog and DVI have different color value, the GAMMA is great to correct the difference |
| Mstr_Ctrl_Disable_Vid = $80 | This will disable the Scanning of the Video hence giving 100% bandwith to the CPU |

MASTER_CTRL_REG_H = $D001

| Mstr_Ctrl_Video_Mode = $01 | 0 - 640x480@60Hz : 1 - 640x400@70hz (text mode) 0 - 320x240@60hz 1 - 320x200@70Hz (Graphic Mode & Text mode when Doubling = 1) |
| --- | --- |
| Mstr_Ctrl_Text_XDouble = $02 | X Pixel Doubling |
| Mstr_Ctrl_Text_YDouble = $04 | Y Pixel Doubling |
| Mstr_Ctrl_Turn_Sync_Off = $08 | 1 = Turn off Sync |
| Mstr_Ctrl_Show_BG_InOverlay = $10 | 1 = Allow the Background color to show up in Overlay mode |
| Mstr_Ctrl_FONT_Bank_Set = $20 | 0 =(default) FONT Set 0, 1 = FONT Set 1 |
| Mstr_Ctrl_Text_MEM_Enable = $40 |  |
| Mstr_Ctrl_Text_MEM_BG_En = $80 |  |

MASTER_CTR_REG_2 = $D00A

| HW_Line_Drawing_Enable = $01 | 0 - Disable Line drawing (This also enables +64 Additional Sprites 1 – Enable HW Line drawing (Disables Additional Sprites) |
| --- | --- |

NEW MEMORY RELOCATABLE TEXT MODE

NEW CHARACTER ATTRIBUTES

HARDWARE BRESENHAM LINE DRAWING

FLOATING POINT BLOCK

BLOCK DIAGRAM FOR FLOAT COPROCESSOR

$DE80: Control Register

| Bit[0] = MUXA Ctrl | 0 = User Input0 1 = Converted User Input0 (Fixed [20:12] -> Float 32bits) |
| --- | --- |
| Bit[1] = MUXB Ctrl | 0 = User Input1 1 = Converted User Input1 (Fixed [20:12] -> Float 32bits) |
| Bit[2] = Not Used |  |
| Bit[3] = Addition Block TValid** |  |
| Bit[5:4] = MUX C Ctrl | 00: Output of MUXA 01: Output of MUXB 10: Mutiplier Output 11: Divider Output |
| Bit[7:6] = Mux D Ctrl | 00: Output of MUXA 01: Output of MUXB 10: Multiplier Output 11: Divider Output |

$DE81: Control Register1

| Bit[1:0]: MUXE Ctrl | 00: Multiplier Output 01: Divider Output 2'b10: Adder Output 11: Float Value of 0 (0x3F80_0000) |
| --- | --- |

$DE82: Control Register2

| Bit[0] | Converter 0 TValid |
| --- | --- |
| Bit[1] | Input 0 TValid |
| Bit[2] | Converter 1 TValid |
| Bit[3] | Input 1 TValid |

DATA REGISTERS  FOR FLOAT OPERATIONS

$DE83: Not Used

//Write

$DE87:$DE84:	 	Not Used (Write Only)

$DE8B:$DE88:		USER Input0 (Little Endian) (Write Only)

$DE8F:$DE8C:		USER Input 1 (Little Endina) (Write Only)

// Read

$DE80: 		Control Register 0

$DE81: 		Control Register 1

$DE82: 		Control Register 2

$DE83: 		Control Register 3

$DE84: 		Status Register 0 (to be detailed Later)

$DE85: 		Status Register 1  (to be detailed Later)

$DE86: 		Status Register 2  (to be detailed Later)

$DE87: 		Status Register 3  (to be detailed Later)

$DE8B:$DE88: 	FLOAT OUTPUT

$DE8F:$DE8C: 	CONVERTED OUTPUT [20:12] Format S+19.12

CHANGES TO DMA CONTROLLER

VDMA_CONTROL_REG        = $DF00

| VDMA_CTRL_Enable [0]= $01 | 0: Disable DMA 1: Enable DMA |
| --- | --- |
| VDMA_CTRL_1D_2D [1]= $02 | 0: 1D (Linear) Transfer 1: 2D (Block) Transfer |
| VDMA_CTRL_TRF_Fill [2]= $04 | 0: Transfer Src -> Dst 1: Fill Destination with "Byte2Write" |
| VDMA_CTRL_Int_Enable [3]= $08 | 0: No Interrupts Generated 1: Enable the Generation of Interrupt when the Transfer is over. |
| NOT USED [4]= $10 | Reserved |
| NOT USED [5]= $20 | Reserved |
| SixteenBit_Enable [6]= $40 | 0: 8 bit transfers 1: to Do a 16 bits Transfer (Even TO Even Address Only) |
| VDMA_CTRL_Start_TRF [7]= $80 | 0: Clear - Needs to be Cleared before you can start a transfer 1: Begin Process |